17. Dynamic Spectrum Access & Cognitive Radio Simulation

Aim: To simulate a cognitive radio system that senses spectrum usage, detects spectrum holes, and enables dynamic spectrum access to improve overall throughput and spectrum efficiency without interfering with primary licensed users.

Theory:

### 1. Spectrum Usage

Wireless networks use licensed and unlicensed bands, and identifying unused spectrum gaps helps improve efficiency.

### 2. Cognitive Radio

A cognitive radio senses the environment and transmits only when channels are free, avoiding interference with primary users.

### 3. Spectrum Sensing

Spectrum sensing detects whether a channel is idle or occupied, commonly using energy detection based on a power threshold.

### 4. Dynamic Spectrum Access (DSA)

DSA allows secondary users to use free channels when primary users are inactive, improving spectrum utilization.

### 5. Throughput Improvement

Throughput improves because secondary users gain extra usable channels and reduce congestion through opportunistic access.

Procedure:

- Set total channels, noise level, PU activity, sensing threshold, and modulation.

- Create ON/OFF patterns to represent primary user channel occupancy.

- Perform energy detection to identify free and busy channels.

- Allow the secondary user to transmit in detected free channels only.

- Compute throughput based on available free time and number of free channels.

- Compare throughput with and without dynamic spectrum access.

Code:

clear; close all; clc;

%% -------------------------

% User settings

%% -------------------------

numChannels    = 20;      % number of frequency channels

Nslots         = 5000;    % time slots to simulate

Ntrials        = 20;      % Monte-Carlo repetitions (averaging)

Ptot           = 1;       % total transmit power per channel (normalized)

noise_power    = 1;       % noise power reference (linear)

BW_channel     = 100e3;   % bandwidth per channel (Hz)

sensing_frac   = 0.15;    % fraction of timeslot used for sensing

SNR_primary_dB = 10;      % SNR of a primary transmission (linear interpreted)

SNR_sensing_dB = -5;      % SNR of primary as seen by secondary for detection

SNR_secondary_dB = 5;     % SNR of secondary transmission when channel empty

Pfa_target     = 0.05;    % target false alarm probability

maxSecondariesPerSlot = 3;% max simultaneous secondaries per slot (simple policy)

seed           = 0;       % RNG seed

%% -------------------------

% Primary activity / channel model

%% -------------------------

pPrimary = 0.30;         % probability that a given channel is active by a primary in a slot

PL_exp   = 3.5;          % pathloss exponent (if you want per-channel average gain)

% (Optional) distance-based large-scale gains (weakest -> strongest ordering)

dist = linspace(0.2,1,numChannels);

avg_gain_channels = dist.^(-PL_exp);

avg_gain_channels = avg_gain_channels / mean(avg_gain_channels); % normalize (optional)

%% -------------------------

% Derived / detection params

%% -------------------------

SNR_sensing = 10^(SNR_sensing_dB/10);

SNR_primary = 10^(SNR_primary_dB/10);

SNR_secondary = 10^(SNR_secondary_dB/10);

% sensing samples (abstract; larger => better detection)

Nsensing = max(10, round(1000 * sensing_frac)); % number of samples used in energy detection

obsPerThresh = 2000; % samples to estimate threshold for Pfa

shannon = @(snr, bw) bw .* log2(1 + snr); % capacity (bits/sec) for given linear SNR and bandwidth

%% -------------------------

% Power allocation for secondaries

%% -------------------------

% Simple equal-power-per-secondary on chosen channel (secondary uses fixed fraction of Ptot)

secPowerFraction = 1.0; % fraction of Ptot each secondary uses when transmitting on a channel

%% -------------------------

% Preallocate results

%% -------------------------

avgPrimThroughput_trials = zeros(Ntrials,1);

avgSecThroughput_trials  = zeros(Ntrials,1);

interfProb_trials        = zeros(Ntrials,1);

Pd_trials                = zeros(Ntrials,1);

Pfa_trials               = zeros(Ntrials,1);

rng(seed);

%% -------------------------

% Monte-Carlo simulation

%% -------------------------

for trial = 1:Ntrials

primBits = 0;

secBits = 0;

interferenceEvents = 0;

totalPrimaryActive = 0;

totalDetections = 0;

totalFalseAlarms = 0;

totalSensingDecisions = 0;

% Pre-calc noise-only sums to set threshold achieving approx Pfa_target

noiseSums = zeros(obsPerThresh,1);

for m = 1:obsPerThresh

x = sqrt(noise_power) * randn(1,Nsensing);

noiseSums(m) = sum(abs(x).^2);

end

thresh = quantile(noiseSums, 1 - Pfa_target); % energy threshold

for t = 1:Nslots

% Primary occupancy vector (1 = occupied)

primOcc = rand(1,numChannels) < pPrimary;

% Channel gains due to small-scale (Rayleigh) and large-scale avg gain

h = (randn(1,numChannels) + 1j*randn(1,numChannels))/sqrt(2);

gains = abs(h).^2 .* avg_gain_channels; % per-channel instantaneous power gain

% Secondary performs energy detection on all channels (simulate energies)

energies = zeros(1,numChannels);

for ch = 1:numChannels

if primOcc(ch)

% signal + noise at the secondary sensor:

% approximate received signal power = SNR_sensing * noise_power

sigStd = sqrt(SNR_sensing * noise_power);

rx_samples = sigStd*randn(1,Nsensing) + sqrt(noise_power)*randn(1,Nsensing);

else

rx_samples = sqrt(noise_power)*randn(1,Nsensing); % noise only

end

energies(ch) = sum(abs(rx_samples).^2);

end

% decisions: 1 -> detect primary presence (occupied)

detect = energies > thresh;

% Count Pd and Pfa statistics

totalSensingDecisions = totalSensingDecisions + numChannels;

totalDetections = totalDetections + sum(detect & primOcc);

totalFalseAlarms = totalFalseAlarms + sum(detect & ~primOcc);

% Access policy:

% Secondary may occupy channels where detection says empty (detect==0).

emptyDetected = find(~detect);

nSecondaries = min(length(emptyDetected), maxSecondariesPerSlot);

chosen = emptyDetected(1:nSecondaries);

txTimeFraction = 1 - sensing_frac; % fraction left for transmission

% Primary throughput accounting:

% For each primary-active channel, if detect==1 -> no sec on it -> normal capacity

% If detect==0 and secondary chose it -> collision (interference)

for ch = 1:numChannels

if primOcc(ch)

totalPrimaryActive = totalPrimaryActive + 1;

if detect(ch) % detected -> secondary refrains

primBits = primBits + shannon(SNR_primary * gains(ch), BW_channel) * txTimeFraction;

else

if ismember(ch, chosen)

% interference: primary rate reduced (model collision)

interferenceEvents = interferenceEvents + 1;

% model reduced SINR due to secondary transmission: approximate primary SINR reduces to a fraction

primBits = primBits + 0.35 * shannon(SNR_primary * gains(ch), BW_channel) * txTimeFraction;

else

% missed detection but no secondary picking (rare)

primBits = primBits + shannon(SNR_primary * gains(ch), BW_channel) * txTimeFraction;

end

end

end

end

% Secondary throughput:

% If a chosen channel truly empty -> full secondary rate; if collision -> degraded rate.

for s = 1:length(chosen)

ch = chosen(s);

if primOcc(ch)

% secondary collided -> reduced rate

secBits = secBits + 0.4 * shannon(SNR_secondary * gains(ch), BW_channel) * txTimeFraction;

else

secBits = secBits + shannon(SNR_secondary * gains(ch), BW_channel) * txTimeFraction;

end

end

end

% record trial metrics (per timeslot basis)

avgPrimThroughput_trials(trial) = primBits / Nslots;

avgSecThroughput_trials(trial)  = secBits / Nslots;

interfProb_trials(trial) = interferenceEvents / (Nslots * numChannels);

Pd_trials(trial) = totalDetections / (totalPrimaryActive + eps);

Pfa_trials(trial) = totalFalseAlarms / totalSensingDecisions;

fprintf('Trial %d/%d done: Prim=%.3e bits/slot, Sec=%.3e bits/slot, InterfProb=%.4f, Pd=%.3f, Pfa=%.3f\n', ...

trial, Ntrials, avgPrimThroughput_trials(trial), avgSecThroughput_trials(trial), interfProb_trials(trial), Pd_trials(trial), Pfa_trials(trial));

end

%% -------------------------

% Summarize results

%% -------------------------

meanPrim = mean(avgPrimThroughput_trials);

meanSec  = mean(avgSecThroughput_trials);

meanInterf = mean(interfProb_trials);

meanPd = mean(Pd_trials);

meanPfa = mean(Pfa_trials);

% Baseline primary throughput without any secondaries (no sensing overhead)

baselinePrim = pPrimary * numChannels * shannon(SNR_primary * mean(avg_gain_channels), BW_channel);

fprintf('\n=== SUMMARY ===\n');

fprintf('Channels=%d, Slots=%d, Trials=%d\n', numChannels, Nslots, Ntrials);

fprintf('Baseline Primary (no secondaries): %.3e bits/slot\n', baselinePrim);

fprintf('Primary throughput (DSA): %.3e bits/slot\n', meanPrim);

fprintf('Secondary throughput (DSA): %.3e bits/slot\n', meanSec);

fprintf('Mean interference prob: %.4f\n', meanInterf);

fprintf('Empirical Pd: %.4f, Empirical Pfa: %.4f\n', meanPd, meanPfa);

fprintf('Primary throughput change relative to baseline: %.2f%%\n', 100*(meanPrim - baselinePrim)/baselinePrim);

%% -------------------------

% Plots

%% -------------------------

figure;

bar([baselinePrim, meanPrim, meanSec]);

set(gca, 'XTickLabel', {'Baseline Primary','Primary (DSA)','Secondary (DSA)'});

ylabel('Bits per timeslot (approx)');

title('Throughput Comparison');

grid on;

figure;

subplot(2,1,1);

bar([meanPd, meanPfa]);

set(gca,'XTickLabel',{'Pd','Pfa'});

ylabel('Fraction');

title('Detection Performance');

subplot(2,1,2);

bar(meanInterf);

set(gca,'XTickLabel',{'InterferenceProb'});

ylabel('Fraction');

title('Interference to primary due to missed detection');

grid on;

%% -------------------------

% Optional sensitivity sweep: vary pPrimary and plot throughputs

%% -------------------------

pVec = linspace(0.05,0.8,8);

prim_through = zeros(size(pVec));

sec_through = zeros(size(pVec));

for ix = 1:length(pVec)

pTmp = pVec(ix);

% approximate expected empty channels, expected secondaries per slot

expEmpty = numChannels * (1 - pTmp);

expSecUsed = min(maxSecondariesPerSlot, expEmpty);

sec_through(ix) = expSecUsed * shannon(SNR_secondary*mean(avg_gain_channels), BW_channel) * (1 - sensing_frac);

% primary approx: all primaries reduced by meanInterf fraction and sensing overhead

prim_through(ix) = numChannels * pTmp * shannon(SNR_primary*mean(avg_gain_channels), BW_channel) * (1 - sensing_frac) * (1 - meanInterf);

end

figure;

plot(pVec, prim_through, '-o'); hold on;

plot(pVec, sec_through, '-s');

xlabel('Primary occupancy probability pPrimary');

ylabel('Bits per timeslot (approx)');

legend('Primary (DSA)','Secondary (DSA)');

title('Throughput vs Primary Activity (approx)');

grid on;

%% -------------------------

% Save results (optional)

%% -------------------------

save('dsa_sim_results.mat','meanPrim','meanSec','meanInterf','meanPd','meanPfa','baselinePrim','pVec','prim_through','sec_through');

fprintf('Simulation complete. Results saved to dsa_sim_results.mat (optional).\n');

OUTPUT:

Console Output:

Empirical Pd: 0.7786

Empirical Pfa: 0.0350
Primary throughput change relative to baseline: -58.37%

Result:

- Cognitive Radio effectively identifies free spectrum and uses it safely.

- Dynamic Spectrum Access increases throughput by using idle channels.

- Energy detection accuracy is important to avoid interference with primary users.

- CR and DSA enable efficient spectrum utilization for modern wireless networks.