16. NOMA Power-Domain Multiplexing Simulation

a) To implement NOMA with 2–3 users sharing the same frequency to study non

orthogonal multiple access performance.

b) To perform successive interference cancellation (SIC) at the receiver to separate

overlapping user signals effectively.

c) To evaluate achievable user rates and power allocation trade-offs to analyze system

fairness and throughput optimization.

AIM: To implement Power-Domain NOMA for 2-3 users sharing the same frequency resource and study non-orthogonal multiple access behaviour, perform Successive Interference Cancellation (SIC) at the receiver to separate superimposed user signals and compute achievable data rates and analyse power allocation trade-offs for improving system fairness and maximizing throughput.

THEORY:

Non-Orthogonal Multiple Access (NOMA)

NOMA allows multiple users to transmit simultaneously on the same time–frequency resource.
Instead of orthogonality (OFDMA), NOMA separates users by power levels:

- Weak users (far from base station) → allocated higher power

- Strong users (near the base station) → allocated lower power

The base station transmits a superposition-coded signal:

x=i=1U1Pisi

where
Pi= allocated power to user i,
si= user symbol.

Achievable Rate

The SINR for user i with remaining interference is:

SINRi=αiP∣hi∣2(k<i1αkP∣hi∣2)+N0

Achievable rate:

Ri=log⁡2(1+SINRi)

Power allocation strongly affects:

- Fairness

- User throughput

- System performance

PROCEDURE:

1. Initialize simulation parameters such as number of users (2 or 3), SNR, noise variance, total transmit power, and number of trials.

2. Generate power allocation combinations (either evenly spaced for 2 users or random normalized vectors for 3 users).

3. For each power allocation, run multiple trials to average the channel behaviour.

4. In each trial, generate fading channel coefficients for all users and compute their channel gains.

5. Sort users by channel strength (weakest to strongest) to establish the SIC decoding order.

6. Compute SINR and achievable rate for each user by considering interference from stronger users that are decoded later.

7. Calculate average user rates, sum rate, and fairness index for each power allocation and identify the configuration with maximum throughput and maximum fairness.

PSEUDO CODE:

clear; clc;

%% Parameters

NumUsers = 3;

numTrials = 5000;

SNR_dB = 20;

SNR = 10^(SNR_dB/10);

noiseVar = 1;

Ptotal = SNR * noiseVar;

if NumUsers == 2

pGrid = linspace(0.1,0.9,30);

powerSets = [pGrid; 1-pGrid]';

else

nAlloc = 500;

raw = rand(nAlloc,3);

powerSets = raw ./ sum(raw,2);

end

numAlloc = size(powerSets,1);

avgRates = zeros(numAlloc,NumUsers);

sumRates = zeros(numAlloc,1);

fairness = zeros(numAlloc,1);

%% Simulation

for idx = 1:numAlloc

p = powerSets(idx,:);

rates_log = zeros(numTrials,NumUsers);

for t = 1:numTrials

h = (randn(NumUsers,1)+1j*randn(NumUsers,1))/sqrt(2);

g = abs(h).^2;

[g_sorted,~] = sort(g,'ascend');

p_sorted = p;

userP = p_sorted * Ptotal;

for u = 1:NumUsers

gu = g_sorted(u);

if u == 1

interf = sum(userP(2:end));

elseif u == NumUsers

interf = 0;

else

interf = sum(userP(u+1:end));

end

SINR = (userP(u)*gu) / (interf*gu + noiseVar);

rates_log(t,u) = log2(1+SINR);

end

end

r = mean(rates_log,1);

avgRates(idx,:) = r;

sumRates(idx) = sum(r);

fairness(idx) = (sum(r))^2 / (NumUsers * sum(r.^2));

end

%% Display values in Command Window

fprintf("\n--------- NOMA SIMULATION OUTPUT ---------\n");

% Print all allocations

for i = 1:numAlloc

fprintf("\nCase %d:\n", i);

fprintf("Power Allocation = ");

disp(powerSets(i,:));

fprintf("Avg Rates = ");

disp(avgRates(i,:));

fprintf("Sum Rate = %.4f\n", sumRates(i));

fprintf("Fairness = %.4f\n", fairness(i));

end

% Best allocations

[~, bestSum] = max(sumRates);

[~, bestFair] = max(fairness);

fprintf("\n===== BEST SUM RATE CONFIGURATION =====\n");

disp(powerSets(bestSum,:));

disp(avgRates(bestSum,:));

fprintf("Sum Rate = %.4f\nFairness = %.4f\n", sumRates(bestSum), fairness(bestSum));

fprintf("\n===== MOST FAIR CONFIGURATION =====\n");

disp(powerSets(bestFair,:));

disp(avgRates(bestFair,:));

fprintf("Sum Rate = %.4f\nFairness = %.4f\n\n", sumRates(bestFair), fairness(bestFair));

OUTPUTS:

RESULT: The NOMA simulation shows that different power allocations create a trade-off between maximizing total throughput and ensuring fairness among users. 
The best-sum-rate allocation gives highest system capacity, while another allocation provides maximum fairness after applying SIC.