13EXP: Beamforming & Beamsteering Simulation

a) To implement analog, digital, and hybrid beamforming for a uniform linear antenna

array to study different beamforming techniques.

b) To visualize radiation patterns and beam direction to analyze how the array focuses

energy toward desired angles.

c) To evaluate beamforming gain and interference reduction to assess the effectiveness

of each beamforming method in improving signal quality.

AIM:

To implement analog, digital, and hybrid beamforming for a Uniform Linear Array (ULA) using MATLAB, visualize radiation patterns, and evaluate beamforming gain and interference suppression.

THEORY:

Beamforming focuses energy in a chosen direction by adjusting antenna weights.

Types of beamforming:

- Analog: Phase-only control using RF phase shifters.

- Digital: Full complex weights applied in baseband (MRC, MVDR).

- Hybrid: Combination of analog phase shifters and few digital RF chains (used in 5G mmWave).

Array factor:Beamforming improves gain, reduces interference, and enhances SINR.

PROCEDURE:

- Set frequency, wavelength, number of antenna elements, spacing, and angle grid in MATLAB.

- Analog beamformer: phase-only weights.

- Digital beamformers: MRC and MVDR formulas.

- Hybrid beamformer: RF matrix + digital baseband weights.

- In MATLAB, plot AF (in dB) for each method to observe beam direction and side-lobes.

- Gain at desired angle

- Gain at interferer angle

- SINR for each beamforming method

- Compare analog vs digital vs hybrid beamforming to identify which gives best gain and interference suppression.

PROGRAM:

% beamforming_sim.m

% Single-file simulation: analog, digital, and hybrid beamforming for a ULA.

% Plots radiation patterns and computes beamforming gain & interference reduction.

% Author: ChatGPT (GPT-5.1)

clear; close all; clc;

%% ========== Simulation Parameters ==========

fc = 3e9;               % carrier frequency (Hz)

c = 3e8;                % speed of light (m/s)

lambda = c/fc;          % wavelength (m)

N = 16;                 % number of antenna elements (ULA)

d = lambda/2;           % element spacing

angles = -90:0.1:90;    % angle grid

k = 2*pi/lambda;

% Desired & interferer angles

theta_des = 20;

theta_int = -30;

% Signal / noise parameters

Ptx = 1;

sigma2 = 1e-3;

Pint = 1;

% Hybrid beamforming

NRF = 3;

maxOMPiter = NRF;

% Convert to radians

angles_rad = deg2rad(angles);

theta_des_rad = deg2rad(theta_des);

theta_int_rad = deg2rad(theta_int);

%% ========== Helper functions ==========

steeringVector = @(N, d, k, theta) exp(1j * k * d * (0:(N-1)).' * sin(theta));

arrayFactor = @(w, N, d, k, angles_rad) ...

arrayfun(@(th) w(:)' * steeringVector(N, d, k, th), angles_rad);

normc = @(x) x / norm(x);

%% ========== Steering vectors ==========

a_des = steeringVector(N, d, k, theta_des_rad);

a_int = steeringVector(N, d, k, theta_int_rad);

%% ========== DIGITAL BEAMFORMING ==========

% 1) MRC (conjugate)

w_dig_mrc = conj(a_des);

w_dig_mrc = w_dig_mrc / norm(w_dig_mrc);

% 2) MVDR

R_int = Pint*(a_int*a_int') + sigma2*eye(N);

w_dig_mvdr = (R_int \ a_des) / (a_des' * (R_int \ a_des));

w_dig_mvdr = w_dig_mvdr / norm(w_dig_mvdr);

%% ========== ANALOG (PHASE-ONLY) BEAMFORMING ==========

w_analog = exp(-1j * angle(a_des));

w_analog = w_analog / norm(w_analog);

%% ========== HYBRID BEAMFORMING (OMP-BASED) ==========

w_opt = conj(a_des);

w_opt = w_opt / norm(w_opt);

% dictionary

Ngrid = 181;

dict_angles = linspace(-pi/2, pi/2, Ngrid);

A_dict = zeros(N, Ngrid);

for ii = 1:Ngrid

A_dict(:,ii) = steeringVector(N,d,k,dict_angles(ii));

end

for ii = 1:Ngrid

A_dict(:,ii) = A_dict(:,ii)/norm(A_dict(:,ii));

end

% OMP

residual = w_opt;

selected_idx = [];

F_RF = zeros(N, NRF);

for iter = 1:maxOMPiter

projections = abs(A_dict' * residual);

[~, idx] = max(projections);

selected_idx = [selected_idx idx];

F_RF(:,iter) = exp(1j * angle(A_dict(:,idx)));

Fsub = F_RF(:,1:iter);

fBB = pinv(Fsub) * w_opt;

residual = w_opt - Fsub * fBB;

end

% Final hybrid

fBB = pinv(F_RF) * w_opt;

W_hybrid = F_RF*fBB;

W_hybrid = W_hybrid / norm(W_hybrid);

%% ========== Array Factors ==========

AF_analog = arrayFactor(w_analog, N, d, k, angles_rad).';

AF_dig_mrc = arrayFactor(w_dig_mrc, N, d, k, angles_rad).';

AF_dig_mvdr = arrayFactor(w_dig_mvdr, N, d, k, angles_rad).';

AF_hybrid = arrayFactor(W_hybrid, N, d, k, angles_rad).';

to_dB = @(x) 20*log10(abs(x)/max(abs(x))+1e-12);

AF_analog_dB = to_dB(AF_analog);

AF_dig_mrc_dB = to_dB(AF_dig_mrc);

AF_dig_mvdr_dB = to_dB(AF_dig_mvdr);

AF_hybrid_dB = to_dB(AF_hybrid);

%% ========== Plot Radiation Patterns ==========

figure('Name','Radiation Patterns','Position',[100 100 1100 600]);

subplot(2,2,1); plot(angles,AF_analog_dB); grid on; ylim([-60 0]);

title('Analog Beamforming'); xlabel('Angle (deg)'); ylabel('Array Pattern (dB)')

subplot(2,2,2); plot(angles,AF_dig_mrc_dB); grid on; ylim([-60 0]);

title('Digital MRC'); xlabel('Angle (deg)'); ylabel('Array Pattern (dB)')

subplot(2,2,3); plot(angles,AF_dig_mvdr_dB); grid on; ylim([-60 0]);

title('Digital MVDR'); xlabel('Angle (deg)'); ylabel('Array Pattern (dB)')

subplot(2,2,4); plot(angles,AF_hybrid_dB); grid on; ylim([-60 0]);

title('Hybrid Beamforming'); xlabel('Angle (deg)'); ylabel('Array Pattern (dB)')

figure('Name','Comparison','Position',[150 150 800 420]);

plot(angles,AF_analog_dB,'LineWidth',1.2); hold on;

plot(angles,AF_dig_mrc_dB,'LineWidth',1.2);

plot(angles,AF_dig_mvdr_dB,'LineWidth',1.2);

plot(angles,AF_hybrid_dB,'LineWidth',1.2);

grid on; xlim([-90 90]);

xlabel('Angle (deg)'); ylabel('dB');

legend('Analog','Digital-MRC','Digital-MVDR','Hybrid','Location','southwest');

title('Comparison of Beamforming Techniques');

%% ========== Beamforming Gain & Interference ==========

resp = @(w,theta) w' * steeringVector(N,d,k,theta);

% FIXED field names (no hyphens)

methods = {'Analog','Digital_MRC','Digital_MVDR','Hybrid'};

weights = {w_analog, w_dig_mrc, w_dig_mvdr, W_hybrid};

results = struct();

for i = 1:length(methods)

w = weights{i};

g_des = abs(resp(w,theta_des_rad))^2;

g_int = abs(resp(w,theta_int_rad))^2;

Psig = Ptx*g_des;

Pint_eff = Pint*g_int;

Pnoise = sigma2*(norm(w)^2);

SINR = Psig/(Pint_eff+Pnoise);

results.(methods{i}).g_des = g_des;

results.(methods{i}).g_int = g_int;

results.(methods{i}).Psig = Psig;

results.(methods{i}).Pint_eff = Pint_eff;

results.(methods{i}).Pnoise = Pnoise;

results.(methods{i}).SINR = SINR;

end

%% ========== Display Results ==========

T = table(methods', ...

cellfun(@(m) results.(m).g_des, methods)', ...

cellfun(@(m) results.(m).g_int, methods)', ...

cellfun(@(m) results.(m).SINR, methods)', ...

'VariableNames', {'Method','ArrayGain_Desired','ArrayGain_Interferer','SINR_Linear'});

disp('--- Beamforming Performance ---');

disp(T);

fprintf("\nSINR (dB):\n");

for i=1:length(methods)

fprintf("%s : %.2f dB\n", methods{i}, 10*log10(results.(methods{i}).SINR));

end

PSEUDO CODE:

clc; clear; close all;

%% PARAMETERS

fc = 28e9;               % 28 GHz

c = 3e8;

lambda = c/fc;

N = 8;                   % Number of antennas

d = lambda/2;            % Element spacing

angles = -90:0.1:90;     % Scan grid (deg)

ang_rad = deg2rad(angles);

desired_ang = 20;        % Desired user

interf_ang = -40;        % Interference

desired_rad = deg2rad(desired_ang);

interf_rad  = deg2rad(interf_ang);

noise = 0.01;

Pint  = 1;

Ptx   = 1;

%% FUNCTION: Steering vector

steer = @(theta) exp(1j*2*pi*(0:N-1)'*d*sin(theta)/lambda);

a_des = steer(desired_rad);

a_int = steer(interf_rad);

%% DIGITAL BEAMFORMING

% ----- MRC -----

w_MRC = conj(a_des);

w_MRC = w_MRC / norm(w_MRC);

% ----- MVDR -----

R = Pint*(a_int*a_int') + noise*eye(N);

w_MVDR = (R \ a_des) / (a_des' * (R \ a_des));

w_MVDR = w_MVDR / norm(w_MVDR);

%% ANALOG BEAMFORMING (phase shifters)

w_analog = exp(1j * angle(a_des));

w_analog = w_analog / norm(w_analog);

%% HYBRID BEAMFORMING (simple RF + BB)

NRF = 2;

F_RF = zeros(N, NRF);

theta_dict = deg2rad(-90:1:90);

A_dict = zeros(N, length(theta_dict));

for i = 1:length(theta_dict)

A_dict(:, i) = steer(theta_dict(i));

end

residual = a_des;

for k = 1:NRF

corr = abs(A_dict' * residual);

[~, idx] = max(corr);

F_RF(:, k) = exp(1j * angle(A_dict(:, idx)));  % phase-only RF

% baseband weights using least squares

fBB = pinv(F_RF) * a_des;

hybrid = F_RF * fBB;

residual = a_des - hybrid;

end

W_hybrid = F_RF * fBB;

W_hybrid = W_hybrid / norm(W_hybrid);

%% ARRAY FACTOR FOR ALL

AF_analog = zeros(size(ang_rad));

AF_MRC    = zeros(size(ang_rad));

AF_MVDR   = zeros(size(ang_rad));

AF_hybrid = zeros(size(ang_rad));

for i = 1:length(ang_rad)

a_theta = steer(ang_rad(i))

AF_analog(i) = abs(w_analog' * a_theta);

AF_MRC(i)    = abs(w_MRC' * a_theta);

AF_MVDR(i)   = abs(w_MVDR' * a_theta);

AF_hybrid(i) = abs(W_hybrid' * a_theta);

end

AF_analog = 20*log10(AF_analog/max(AF_analog));

AF_MRC    = 20*log10(AF_MRC/max(AF_MRC));

AF_MVDR   = 20*log10(AF_MVDR/max(AF_MVDR));

AF_hybrid = 20*log10(AF_hybrid/max(AF_hybrid));

%% PLOTS

figure;

plot(angles,AF_analog,'LineWidth',2); hold on;

plot(angles,AF_MRC,'LineWidth',2);

plot(angles,AF_MVDR,'LineWidth',2);

plot(angles,AF_hybrid,'LineWidth',2);

grid on; xlabel('Angle (deg)'); ylabel('Array Factor (dB)');

title('Beamforming Patterns');

legend('Analog','MRC','MVDR','Hybrid');

%% GAIN & SINR

methods = {'Analog','MRC','MVDR','Hybrid'};

weights = {w_analog, w_MRC, w_MVDR, W_hybrid};

disp('--- Beamforming Performance ---');

for i = 1:4

w = weights{i};

g_des = abs(w' * a_des)^2;

g_int = abs(w' * a_int)^2;

Psig = Ptx * g_des;

Pint_eff = Pint * g_int;

Pnoise = noise * norm(w)^2;

SINR = Psig / (Pint_eff + Pnoise);

fprintf("%s: Gain(des)=%.2f  Gain(int)=%.2f  SINR(dB)=%.2f\n", ...

methods{i}, g_des, g_int, 10*log10(SINR));

end

OUTPUT:

OUTPUT CONSOLE:

Beamforming Performance ---

Method         ArrayGain_Desired    ArrayGain_Interferer    SINR_Linear
    ________________    _________________    ____________________    ___________

    {'Analog'      }       0.080218                   0.563           0.14223  
    {'Digital_MRC' }        0.080218                   0.563           0.14223  
    {'Digital_MVDR'}                      15.964                                                       1.4149e-10                       15964

{'Hybrid'      }                              0.080218                   	                         0.563                              0.14223

SINR (dB):
Analog : -8.47 dB
Digital_MRC : -8.47 dB
Digital_MVDR : 42.03 dB
Hybrid : -8.47 dB

CONCLUSION:

Beamforming techniques successfully steer the beam toward the desired direction and reduce interference. Digital and hybrid beamforming provide higher gain and better interference suppression compared to analog beamforming, making them more suitable for advanced wireless systems.