4. Hexagonal Cell Structure &amp; Capacity Planning Simulation

AIM:

To simulate a hexagonal cellular network structure and analyze its coverage, frequency reuse, and capacity performance by calculating cell radius, coverage area, user distribution, and the effect of cluster size on overall network capacity.

TOOLS REQUIRED:

- MATLAB

- Communications Toolbox

- Signal Processing Toolbox

- Laptop/PC

THEORY:

• Hexagonal Cell Structure:
Cellular networks commonly use a hexagonal geometry to model coverage regions. The hexagon shape is preferred because it provides uniform coverage over a plane without gaps or overlaps and simplifies frequency planning.

○ Cell Area:
The area of a single hexagonal cell with radius R is given by:

where R is the cell radius.

• Frequency Reuse:
To increase spectrum efficiency, cellular networks apply the concept of frequency reuse, where the same frequency channels are allocated to cells that are sufficiently far apart to avoid interference.
The cluster size (K) represents the number of cells in a group using unique frequencies. Larger K reduces interference but decreases capacity.

• Capacity Planning:
Capacity planning determines how many users or channels a cellular network can support.

○ Channels per Cell:

A smaller cluster size increases channels per cell, thus improving system capacity.

PROCEDURE:

- Set the simulation parameters in MATLAB, including cell radius, number of tiers, bandwidth, spectral efficiency, and cluster size (K).

- Generate the hexagonal cell centers and plot the 7-cell hexagonal structure using the given radius.

- Apply frequency reuse by assigning colors to cells based on the reuse factor K.

- Randomly distribute users within the coverage region and assign each user to the nearest cell.

- Calculate coverage area, bandwidth per cell, capacity per cell, and users per cell based on simulation outputs.

- Plot the user distribution and capacity variation for different cluster sizes.

- Tabulate and record the results obtained from the simulation.

CODE :

%% ===============================================================

%  Hexagonal Cell Structure & Capacity Planning Simulation

%  AIM:

%  1. Simulate hexagonal cellular structure

%  2. Calculate coverage area, cell radius, & user capacity

%  3. Visualize frequency reuse patterns & user distribution

%  4. Analyze effect of frequency reuse (K) on network capacity

%% ===============================================================

clear; close all; clc;

%% ------------------- Simulation Parameters ---------------------

cellRadius = 500;            % radius in meters

numTiers = 1;                % 1-tier hex = 7 cells

totalUsers = 300;            % total users to place

bandwidth = 10e6;            % 10 MHz BW

spectralEfficiency = 2;      % 2 bps/Hz (avg)

clusterSize_K = 3;           % frequency reuse factor (3, 4, 7, etc.)

reuseColors = ['r','g','b','m','c','y'];

%% -------------------- Derived Parameters ------------------------

coverageArea = (3*sqrt(3)/2) * cellRadius^2;

capacity_per_cell = (bandwidth/clusterSize_K) * spectralEfficiency;

fprintf("----- CELL PARAMETERS -----\n");

fprintf("Cell Radius: %.2f m\n", cellRadius);

fprintf("Coverage Area: %.2f sq.m\n", coverageArea);

fprintf("Bandwidth per cell (Reuse K=%d): %.2f MHz\n", clusterSize_K, bandwidth/1e6/clusterSize_K);

fprintf("Capacity per cell: %.2f Mbps\n", capacity_per_cell/1e6);

fprintf("---------------------------\n");

%% ------------------- Generate Hexagonal Grid -------------------

% hex center offsets

a = cellRadius * 1.5;

b = cellRadius * sqrt(3)/2;

centers = [

0 0;

a  b;

a -b;

-a  b;

-a -b;

0 2*b;

0 -2*b

];

numCells = size(centers,1);

%% ---------------------- Plot Hex Cells -------------------------

figure; hold on; axis equal;

title("Hexagonal Cellular Network with Frequency Reuse");

theta = linspace(0, 2*pi, 7);

for i = 1:numCells

% draw hex

x_hex = centers(i,1) + cellRadius*cos(theta);

y_hex = centers(i,2) + cellRadius*sin(theta);

% assign frequency color based on K reuse

color = reuseColors(mod(i, clusterSize_K) + 1);

patch(x_hex, y_hex, color, 'FaceAlpha',0.2, 'EdgeColor','k');

text(centers(i,1), centers(i,2), sprintf("Cell %d", i), ...

'HorizontalAlignment','center', 'FontWeight','bold');

end

legend("Reuse Pattern (K="+string(clusterSize_K)+")");

grid on;

%% ------------------- User Distribution -------------------------

userX = (max(centers(:,1))-min(centers(:,1))) * rand(totalUsers,1) - cellRadius;

userY = (max(centers(:,2))-min(centers(:,2))) * rand(totalUsers,1) - cellRadius;

% assign users to nearest cell

assignedCell = zeros(totalUsers,1);

for u = 1:totalUsers

d = sqrt((userX(u)-centers(:,1)).^2 + (userY(u)-centers(:,2)).^2);

[~, assignedCell(u)] = min(d);

end

plot(userX, userY, 'ko', 'MarkerSize',3);

%% -------- Count users per cell & compute capacity ---------------

usersPerCell = zeros(numCells,1);

for i = 1:numCells

usersPerCell(i) = sum(assignedCell == i);

end

perUserCapacity = capacity_per_cell ./ max(usersPerCell,1);

%% --------------------- Plot Users Per Cell ----------------------

figure;

bar(usersPerCell);

xlabel("Cell Index");

ylabel("Number of Users");

title("User Distribution per Cell");

grid on;

%% -------------------- Capacity vs Cluster Size ------------------

K_values = [1 3 4 7];

capacity_vals = bandwidth./K_values * spectralEfficiency;

figure;

plot(K_values, capacity_vals/1e6, '-o', 'LineWidth',2);

xlabel("Frequency Reuse Factor (Cluster Size K)");

ylabel("Per-Cell Capacity (Mbps)");

title("Effect of Frequency Reuse on Capacity");

grid on;

OUTPUT:

TABEL:

| Cell No: | X(km) | Y(km) | Num users | Channels per cell | Max users cell |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 |  | 3.33 MHz |  |
| 2 | 0.750 | 0.433 |  | 3.33 MHz |  |
| 3 | 0.750 | -0.433 |  | 3.33 MHz |  |
| 4 | -0.750 | 0.433 |  | 3.33 MHz |  |
| 5 | -0.750 | -0.433 |  | 3.33 MHz |  |
| 6 | 0 | 0.866 |  | 3.33 MHz |  |
| 7 | 0 | -0.866 |  | 3.33 MHz |  |

Channels per Cell
From your code:

Bandwidth = 10 MHz

Cluster size K = 3

BW per cell = 10 / 3 = 3.33 MHz

RESULT:

Hexagonal cells provide efficient coverage with minimal gaps, and frequency reuse improves network capacity. Simulation of user distribution and cluster layouts highlights the need for capacity planning and load balancing.