18. mmWave Path Loss & Beamforming Simulation

Objective:

- Simulate mmWave propagation at 28 GHz to study high-frequency signal behaviour (free-space loss, penetration loss, and link budget).

- Compute and compare free-space path loss (FSPL), material penetration loss, and resulting received power for a range of distances.

Theory:

- mmWave (28 GHz) signals have very small wavelength, leading to high free-space path loss.

- Free-Space Path Loss (FSPL):

FSPL (dB)=20log⁡10(4πdλ)\text{FSPL(dB)} = 20\log_{10}\left(\frac{4\pi d}        {\lambda}\right)FSPL(dB)=20log10 (λ4πd )

- mmWave signals suffer large penetration loss through walls, glass, and obstacles.

- Beamforming uses multiple antenna elements to focus energy, increasing gain and coverage.

- Uniform Linear Array (ULA) creates directional beams by applying phase shifts to each element.

- Array Gain ≈

10log⁡10(N) dB10\log_{10}(N) \text{ dB}10log10 (N) dB

(N = number of elements)

Procedure:

- Set frequency = 28 GHz, compute wavelength.

- Define distances (e.g., 1–1000 m) for evaluating path loss.

- Calculate free-space path loss using FSPL formula.

- Add penetration loss (glass, wall, etc.) if modeling indoor/outdoor cases.

- Create a Uniform Linear Array (ULA) with N elements.

- Set element spacing = λ/2.

- Apply phase shifts to steer beam toward desired angle.

- Compute array factor and radiation pattern across angles.

- Convert radiation pattern to gain (dB).

- Combine FSPL, array gain, and Tx power to compute received power.

Code:

% MM-WAVE SIMULATION WITH CONSOLE OUTPUT (28 GHz)

clear; clc; close all;

%% (a) mmWave Propagation at 28 GHz fc = 28e9; c = 3e8; lambda = c/fc;

d = 1:1:200; % distance (m) FSPL = 20log10(4pi*d/lambda);

%% (b) FSPL + Penetration Loss + Beamforming Gain materialLoss = 6; % (dB) one wall N = 8; % ULA elements BF_gain = 10*log10(N); % beamforming gain (dB)

TotalLoss = FSPL + materialLoss - BF_gain;

% Example link budget Ptx_dBm = 20; % 20 dBm = 100 mW Pr_dBm = Ptx_dBm - TotalLoss;

% Noise (simple) Noise_dBm = -90; % assume -90 dBm noise SNR_dB = Pr_dBm - Noise_dBm;

%% (c) Antenna Radiation Pattern theta = -90:0.1:90; theta_rad = deg2rad(theta);

AF = abs( sin(Npisin(theta_rad)) ./ (Nsin(pisin(theta_rad))) ); AF_dB = 20*log10(AF/max(AF));

%% ====== PLOTS ====== figure; plot(d, FSPL, 'LineWidth', 1.5); title('FSPL at 28 GHz'); xlabel('Distance (m)'); ylabel('FSPL (dB)'); grid on;

figure; plot(d, TotalLoss, 'r', 'LineWidth',1.5); title('Total Link Loss'); xlabel('Distance (m)'); ylabel('Loss (dB)'); grid on;

figure; plot(theta, AF_dB, 'LineWidth',1.5); title('8-Element ULA Beam Pattern'); xlabel('Angle (deg)'); ylabel('Gain (dB)'); grid on;

%% ====== CONSOLE OUTPUT ====== fprintf('\n--- MM-WAVE SIMULATION SUMMARY ---\n'); fprintf('Frequency: %.1f GHz\n', fc/1e9); fprintf('Wavelength: %.3f mm\n', lambda*1e3); fprintf('Beamforming gain (8-element ULA): %.2f dB\n', BF_gain); fprintf('Penetration loss used: %.1f dB\n\n', materialLoss);

fprintf('At 10 m:\n'); idx10 = 10; fprintf(' FSPL = %.2f dB\n', FSPL(idx10)); fprintf(' Total Loss = %.2f dB\n', TotalLoss(idx10)); fprintf(' Received Power = %.2f dBm\n', Pr_dBm(idx10)); fprintf(' SNR = %.2f dB\n\n', SNR_dB(idx10));

fprintf('At 100 m:\n'); idx100 = 100; fprintf(' FSPL = %.2f dB\n', FSPL(idx100)); fprintf(' Total Loss = %.2f dB\n', TotalLoss(idx100)); fprintf(' Received Power = %.2f dBm\n', Pr_dBm(idx100)); fprintf(' SNR = %.2f dB\n', SNR_dB(idx100));

fprintf('\nSimulation complete.\n');

OUTPUT:

CONSOLE OUTPUT:

--- MM-WAVE SIMULATION SUMMARY ---
Frequency: 28.0 GHz
Wavelength: 10.714 mm
Beamforming gain (8-element ULA): 9.03 dB
Penetration loss used: 6.0 dB

At 10 m:
 FSPL = 81.38 dB
 Total Loss = 78.35 dB
 Received Power = -58.35 dBm
 SNR = 31.65 dB

At 100 m:
 FSPL = 101.38 dB
 Total Loss = 98.35 dB
 Received Power = -78.35 dBm
 SNR = 11.65 dB

Simulation complete.

Conclusion:

- mmWave signals at 28 GHz experience very high free-space path loss, especially as distance increases.

- Because of the short wavelength, mmWave signals show poor penetration, and obstacles like walls cause large additional losses.

- Beamforming using a multi-element antenna array provides significant array gain, improving received power and extending coverage.

- Directional beams greatly focus energy, increasing link reliability in desired directions.